HVAC SCOPE OF WORK This document defines the complete scope, specifications, standards, and pricing for HVAC work described herein. All work is subject to Client authorization and site verification.

📋  SECTION 1 — PROJECT IDENTIFICATION

| PROJECT INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Name / Title: | ________________________ |
| SOW Number: | ________________________ |
| Revision #: | Rev. ___ ¦ Date: __________ |
| Project Address: | ________________________ |
| City, State, ZIP: | ________________________ |
| Building Type: | ☐ Commercial ☐ Industrial ☐ Retail ☐ Medical ☐ Residential ☐ Other: ______ |
| Building Sq. Ft.: | ____________ ¦ Stories: ____ ¦ Year Built: ____ |
| Project Type: | ☐ New Install ☐ Replacement ☐ Retrofit ☐ Repair ☐ PM Service ☐ Other: ______ |
| CONTRACTOR INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Company Name: | [Company Name] |
| Project Manager: | ________________________ ¦ Phone: ____________________ |
| Lead Technician: | ________________________ ¦ Cert #: ___________________ |
| Contractor License #: | ________________________ ¦ EPA 608 #: ________________ |
| Insurance Certificate: | ☐ On File ☐ To Be Provided ¦ COI Expiration: __________ |
| CLIENT INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Client / Company Name: | ________________________ |
| Primary Contact: | ________________________ ¦ Title: ____________________ |
| Phone: | ________________________ ¦ Email: ____________________ |
| Billing Address: | ________________________ |
| Authorized Decision Maker: | ________________________ ¦ Phone: ____________________ |
| KEY DATES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SOW Issue Date: | ________________________ |
| Client Authorization Due: | ________________________ |
| Scheduled Start Date: | ________________________ |
| Estimated Completion Date: | ________________________ |
| Site Access Hours: | ________________________ (e.g., Mon–Fri 7AM–5PM) |

🏗️  SECTION 2 — SITE CONDITIONS & PRE-WORK SURVEY

Contractor has performed (or shall perform) a site survey prior to work commencement. The conditions noted below form the basis for the scope and pricing in this document. Changes discovered at time of execution that differ materially from the conditions below may require a Change Order.

| Site Condition | Observation / Notes | Action Required |
| --- | --- | --- |
| Electrical Service Available |  | ☐ Yes ☐ No ☐ TBD |
| Adequate Structural Support |  | ☐ Yes ☐ No ☐ TBD |
| Existing Equipment to Remove |  | ☐ Yes ☐ No ☐ TBD |
| Asbestos / Hazmat Present |  | ☐ Yes ☐ No ☐ TBD |
| Permit Required |  | ☐ Yes ☐ No ☐ TBD |
| Existing Ductwork Usable |  | ☐ Yes ☐ No ☐ TBD |
| Roof / Slab Access Adequate |  | ☐ Yes ☐ No ☐ TBD |
| Confined Space Entry Required |  | ☐ Yes ☐ No ☐ TBD |
| Special PPE or Safety Plan Needed |  | ☐ Yes ☐ No ☐ TBD |
| Utility Shutdowns Required |  | ☐ Yes ☐ No ☐ TBD |

2.1  EXISTING EQUIPMENT TO BE REMOVED / REPLACED

| # | Equipment Type | Make / Model | Serial # | Tonnage/MBH | Location | Condition | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  | ☐ Fair ☐ Poor ☐ Failed | ☐ Salvage ☐ Scrap ☐ Client |
| 2 |  |  |  |  |  | ☐ Fair ☐ Poor ☐ Failed | ☐ Salvage ☐ Scrap ☐ Client |
| 3 |  |  |  |  |  | ☐ Fair ☐ Poor ☐ Failed | ☐ Salvage ☐ Scrap ☐ Client |
| 4 |  |  |  |  |  | ☐ Fair ☐ Poor ☐ Failed | ☐ Salvage ☐ Scrap ☐ Client |
| 5 |  |  |  |  |  | ☐ Fair ☐ Poor ☐ Failed | ☐ Salvage ☐ Scrap ☐ Client |

🔧  SECTION 3 — DETAILED SCOPE OF WORK

The following sections define the complete scope of HVAC work to be performed. Each trade scope is organized by discipline. Items marked INCLUDED are part of the base contract price. Items marked EXCLUDED are specifically not included and require a separate Change Order if desired.

3.1  EQUIPMENT SUPPLY & INSTALLATION

| # | Equipment Description | Make / Model / Part # | Qty | Location | Incl? | Notes / Specs |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Rooftop Unit (RTU) — Packaged Gas/Electric |  | 1 | Roof — Unit A | ☐ Y ☐ N |  |
| 2 | Air Handler Unit (AHU) |  | 1 | Mechanical Room | ☐ Y ☐ N |  |
| 3 | Condensing Unit / Heat Pump |  | 1 |  | ☐ Y ☐ N |  |
| 4 | Indoor Evaporator Coil |  | 1 |  | ☐ Y ☐ N |  |
| 5 | Gas Furnace / Heating Unit |  | 1 |  | ☐ Y ☐ N |  |
| 6 | Fan Coil Unit (FCU) |  |  |  | ☐ Y ☐ N |  |
| 7 | Exhaust Fan / Ventilation Unit |  |  |  | ☐ Y ☐ N |  |
| 8 | Energy Recovery Ventilator (ERV/HRV) |  |  |  | ☐ Y ☐ N |  |
| 9 | Split System — Mini-Split / VRF |  |  |  | ☐ Y ☐ N |  |
| 10 | Dehumidifier / Humidifier |  |  |  | ☐ Y ☐ N |  |

3.1.1  All equipment shall be new, factory-warranted, and listed by an NRTL (UL, ETL, or equivalent) unless otherwise noted in writing.

3.1.2  Equipment shall be installed per manufacturer's installation instructions, applicable codes, and ASHRAE standards.

3.1.3  Contractor shall provide equipment submittals (cut sheets, performance data) for Client review and approval prior to procurement unless waived in writing.

3.1.4  Equipment startup and commissioning shall be performed by Contractor or manufacturer representative as required by warranty terms.

3.1.5  Client is responsible for providing a level, structurally adequate mounting surface unless structural preparation is explicitly listed as included.

3.2  DUCTWORK, DISTRIBUTION & AIR DEVICES

| # | Description | Size / Spec | Quantity / LF | Incl? | Location / Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Main Supply Trunk — Galvanized Sheet Metal |  |  | ☐ Y ☐ N |  |
| 2 | Branch Supply Runs — Sheet Metal |  |  | ☐ Y ☐ N |  |
| 3 | Flexible Duct — Insulated (R-6 or R-8) |  |  | ☐ Y ☐ N |  |
| 4 | Return Air Plenum / Trunk |  |  | ☐ Y ☐ N |  |
| 5 | Supply Registers / Grilles |  |  | ☐ Y ☐ N |  |
| 6 | Return Air Grilles |  |  | ☐ Y ☐ N |  |
| 7 | Diffusers — Ceiling (Type: __________) |  |  | ☐ Y ☐ N |  |
| 8 | Balancing Dampers |  |  | ☐ Y ☐ N |  |
| 9 | Fire / Smoke Dampers (AHJ-required) |  |  | ☐ Y ☐ N |  |
| 10 | Duct Insulation — External Wrap (R-____) |  |  | ☐ Y ☐ N |  |
| 11 | Duct Liner — Internal (where specified) |  |  | ☐ Y ☐ N |  |
| 12 | Duct Sealing — Mastic or UL-181 Tape |  |  | ☐ Y ☐ N |  |
| 13 | Fresh Air / Outside Air Intake |  |  | ☐ Y ☐ N |  |
| 14 | Exhaust Ductwork |  |  | ☐ Y ☐ N |  |

3.2.1  All sheet metal ductwork shall be fabricated and installed per SMACNA HVAC Duct Construction Standards (latest edition) and ASHRAE 90.1 duct sealing requirements.

3.2.2  Duct leakage testing shall be performed if required by code or owner. Leakage class and test pressure to be specified: Class _____, Test Pressure: _____ in. W.C.

3.2.3  All supply and return plenums, trunks, and branches shall be insulated to minimum R-6 in unconditioned spaces unless noted otherwise.

3.2.4  Flexible duct connections shall not exceed 5 feet in length per ACCA Manual D guidelines. Flex shall be fully extended without kinks or sags.

3.2.5  Fire and smoke dampers shall be installed per NFPA 90A and coordinated with the AHJ inspection. Access doors shall be provided at each damper.

3.3  REFRIGERANT PIPING & LINE SETS

| # | Description | Size / Spec | Length (LF) | Incl? | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Suction Line — ACR Copper, Insulated |  |  | ☐ Y ☐ N | Insulation: R-4 min., 3/4" wall |
| 2 | Liquid Line — ACR Copper |  |  | ☐ Y ☐ N |  |
| 3 | Hot Gas / Discharge Line |  |  | ☐ Y ☐ N |  |
| 4 | Line Set — Pre-charged (mini-split) |  |  | ☐ Y ☐ N |  |
| 5 | Refrigerant — Type & Charge (lbs) |  |  | ☐ Y ☐ N | Refrigerant Type: ____________ |
| 6 | Nitrogen Pressure / Leak Test |  | N/A | ☐ Y ☐ N | Test pressure: 400–500 psig per mfr. |
| 7 | Evacuation — Triple Pull to <500 microns |  | N/A | ☐ Y ☐ N |  |
| 8 | Filter Drier — Bi-Flow / Angle |  |  | ☐ Y ☐ N |  |
| 9 | Service Valves — Schrader / Ball |  |  | ☐ Y ☐ N |  |
| 10 | Line Set Insulation & UV Jacket |  |  | ☐ Y ☐ N | Where exposed to exterior |

3.3.1  All refrigerant work shall be performed by EPA Section 608 Certified technicians in accordance with 40 CFR Part 82. Refrigerant shall never be intentionally vented.

3.3.2  Copper refrigerant piping shall be Type ACR (cleaned and capped), ASTM B280. Joints shall be brazed with nitrogen flowing internally to prevent oxidation scaling.

3.3.3  System shall be pressure tested with dry nitrogen at manufacturer-specified test pressure, held for minimum [15] minutes with no pressure drop before evacuation.

3.3.4  System shall be evacuated with a two-stage vacuum pump to ≤500 microns. Vacuum shall hold for [30] minutes before refrigerant charge is introduced.

3.3.5  Refrigerant charge shall be verified using manufacturer's charging method (superheat, subcooling, or weigh-in as applicable). Charge weight shall be recorded on the equipment data plate.

3.4  ELECTRICAL, CONTROLS & THERMOSTATS

| # | Description | Spec / Rating | Quantity | Incl? | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Dedicated Electrical Circuit — New |  |  | ☐ Y ☐ N | Coordinate with licensed electrician |
| 2 | Disconnect Switch — Non-Fused |  |  | ☐ Y ☐ N | Within sight of equipment |
| 3 | Disconnect Switch — Fused |  |  | ☐ Y ☐ N |  |
| 4 | Low-Voltage Control Wiring (18/5 or 18/8) |  |  | ☐ Y ☐ N |  |
| 5 | Thermostat — Programmable |  |  | ☐ Y ☐ N |  |
| 6 | Thermostat — Smart / Wi-Fi (e.g., Ecobee, Nest) |  |  | ☐ Y ☐ N |  |
| 7 | Thermostat — Commercial / BACnet-Compatible |  |  | ☐ Y ☐ N |  |
| 8 | Zone Control Board & Dampers |  |  | ☐ Y ☐ N | # of Zones: _____ |
| 9 | BMS / DDC Integration (points list TBD) |  |  | ☐ Y ☐ N |  |
| 10 | CO / CO2 Sensors |  |  | ☐ Y ☐ N |  |
| 11 | Smoke Detector Interlock (AHU shutdown) |  |  | ☐ Y ☐ N |  |
| 12 | Condensate Overflow Switch / Sensor |  |  | ☐ Y ☐ N |  |
| 13 | Variable Frequency Drive (VFD) |  |  | ☐ Y ☐ N | HP: _____ |
| 14 | Electrical Work by Licensed Electrician |  |  | ☐ Y ☐ N | Sub-contract required |

3.4.1  All electrical work shall comply with the current edition of NFPA 70 (National Electrical Code) and local amendments. Electrical permits and inspections are the responsibility of the licensed electrical contractor performing the work.

3.4.2  HVAC Contractor is responsible for control wiring unless Electrical sub-contractor scope is explicitly defined herein. Line-voltage wiring shall be performed by a licensed electrician unless Contractor holds appropriate electrical license.

3.4.3  Thermostat(s) shall be installed, programmed, and commissioned. Setpoint schedules shall be programmed per Client's schedule or as directed.

3.4.4  BMS integration, if applicable, shall include a points list submitted to and approved by the building controls contractor prior to rough-in.

3.5  CONDENSATE DRAINAGE, GAS PIPING & MECHANICAL

| # | Description | Material / Spec | Quantity / LF | Incl? | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Condensate Primary Drain Line | PVC Sch. 40 |  | ☐ Y ☐ N | Sloped min. 1/8" per ft. |
| 2 | Condensate Secondary / Overflow Line | PVC Sch. 40 |  | ☐ Y ☐ N | Visible drain per code |
| 3 | Condensate Pump (if gravity drain not feasible) |  | 1 | ☐ Y ☐ N |  |
| 4 | Condensate Drain Pan — Auxiliary |  | 1 | ☐ Y ☐ N | Required above finished ceiling |
| 5 | Gas Supply Piping — New Run | Black Steel / CSST |  | ☐ Y ☐ N | Licensed plumber/gas fitter required |
| 6 | Gas Shut-Off Valve — Dedicated |  | 1 | ☐ Y ☐ N | Within 6 ft. of appliance |
| 7 | Flue / Venting — B-Vent, PVC, or SS Liner |  |  | ☐ Y ☐ N | Per appliance mfr. specs |
| 8 | Combustion Air Opening / Louver |  |  | ☐ Y ☐ N | Size per NFPA 54 / IRC |
| 9 | Vibration Isolators / Spring Mounts |  |  | ☐ Y ☐ N |  |
| 10 | Seismic Bracing (if required by code) |  |  | ☐ Y ☐ N | Per SMACNA Seismic Guide |

3.6  START-UP, COMMISSIONING & SYSTEM TESTING

|  | Commissioning Task | Included | Responsible Party |
| --- | --- | --- | --- |
| 1 | Manufacturer start-up / commissioning (per warranty requirement) | ☐ Yes ☐ No | ☐ Contractor ☐ Mfr. Rep |
| 2 | Refrigerant charge verification (superheat / subcooling) | ☐ Yes ☐ No | Contractor |
| 3 | Airflow measurement — supply and return (CFM) | ☐ Yes ☐ No | Contractor |
| 4 | External static pressure measurement | ☐ Yes ☐ No | Contractor |
| 5 | Supply and return air temperature differential (ΔT) | ☐ Yes ☐ No | Contractor |
| 6 | Gas pressure verification — manifold and supply | ☐ Yes ☐ No | Contractor |
| 7 | Electrical — voltage, amperage, and phase checks | ☐ Yes ☐ No | Contractor |
| 8 | Controls sequence of operations verification | ☐ Yes ☐ No | Contractor |
| 9 | Thermostat programming and schedule setup | ☐ Yes ☐ No | Contractor |
| 10 | Zone balancing (if applicable) | ☐ Yes ☐ No | ☐ Contractor ☐ TAB Agency |
| 11 | Building Automation System (BAS) point verification | ☐ Yes ☐ No | ☐ Contractor ☐ Controls Sub |
| 12 | Owner / operator training (min. [1] hour) | ☐ Yes ☐ No | Contractor |
| 13 | Completed start-up report submitted to Client | ☐ Yes ☐ No | Contractor |

🚫  SECTION 4 — EXCLUSIONS — NOT INCLUDED IN THIS SCOPE

The following items are explicitly excluded from this Scope of Work unless added by written Change Order. Exclusions do not imply that the items are unnecessary — they are simply outside the contracted scope and price.

| # | Excluded Item | Notes / Condition |
| --- | --- | --- |
| 1 | Line-voltage electrical work (circuits, panels, breakers) | Requires licensed electrician — not included unless noted in Section 3.4 |
| 2 | Gas piping beyond dedicated shut-off valve | Requires licensed plumber or gas fitter |
| 3 | Structural modifications, roof curb fabrication, or roof patching | Owner / GC responsibility unless specified |
| 4 | Building permit fees and AHJ inspection fees | Billed separately at cost — not in base price |
| 5 | Hazardous material abatement (asbestos, mold, lead) | Requires licensed abatement contractor |
| 6 | TAB (Test, Adjust, Balance) by independent TAB agency | Available as add-on — see Change Order |
| 7 | BAS / DDC programming beyond point connection | Controls sub-contractor scope |
| 8 | Fire alarm integration or sprinkler coordination | Fire protection contractor responsibility |
| 9 | Concrete pads, housekeeping pads, or equipment pads | Site/GC work unless listed in Section 3 |
| 10 | Dumpster, waste hauling, or refrigerant reclaim fees | Billed separately unless noted |
| 11 | Work outside normal business hours unless explicitly included | After-hours rates apply per service agreement |
| 12 | Equipment removal / disposal for items not listed in Section 2.1 | Requires Change Order |
| 13 | Third-party commissioning or owner's commissioning agent | Outside Contractor's scope |
| 14 | As-built drawings / CAD documentation | Available as add-on service |
| 15 | Equipment warranties beyond manufacturer's standard warranty | Extended warranties available separately |

NOTE:  Any item not explicitly listed as INCLUDED in Section 3 is considered EXCLUDED. If a required item is discovered during work that was not reasonably foreseeable during the pre-work survey, Contractor will issue a written Change Order before proceeding.

📐  SECTION 5 — APPLICABLE CODES, STANDARDS & SUBMITTALS

5.1  GOVERNING CODES & STANDARDS

| Standard / Code | Title | Applicability |
| --- | --- | --- |
| IMC / UMC (current edition) | International / Uniform Mechanical Code | Mechanical installation |
| NFPA 70 (NEC) | National Electrical Code | Electrical work |
| NFPA 90A / 90B | Standard for HVAC Systems | Duct construction & fire protection |
| NFPA 54 / NFPA 58 | National Fuel Gas Code / LP Gas Code | Gas piping and venting |
| ASHRAE 62.1 / 62.2 | Ventilation for Acceptable Indoor Air Quality | Ventilation rates |
| ASHRAE 90.1 | Energy Standard for Buildings | Equipment efficiency & duct sealing |
| ACCA Manual J / D / S | Load Calc / Duct Design / Equipment Selection | Residential design |
| SMACNA HVAC Duct Const. | SMACNA Duct Construction Standards | Sheet metal fabrication |
| 40 CFR Part 82 | EPA Section 608 — Refrigerant Regulations | Refrigerant handling |
| Local AHJ Requirements | State & Local Codes (see project address) | All applicable trades |

5.2  SUBMITTALS REQUIRED

| # | Submittal Item | Submitted By | Due Date | Status |
| --- | --- | --- | --- | --- |
| 1 | Equipment cut sheets / product data | Contractor | Prior to order | ☐ Pending ☐ Approved |
| 2 | Equipment submittals — performance data | Contractor | Prior to order | ☐ Pending ☐ Approved |
| 3 | Duct shop drawings (if required) | Contractor | Prior to fabrication | ☐ Pending ☐ Approved |
| 4 | Refrigerant piping layout sketch | Contractor | Prior to rough-in | ☐ Pending ☐ Approved |
| 5 | Electrical single-line (if new service) | Electrical Sub | Prior to rough-in | ☐ Pending ☐ Approved |
| 6 | Controls sequence of operations | Contractor | Prior to controls install | ☐ Pending ☐ Approved |
| 7 | Start-up / commissioning report | Contractor | At completion | ☐ Pending ☐ Submitted |
| 8 | O&M manuals and warranty documentation | Contractor | At completion | ☐ Pending ☐ Submitted |
| 9 | As-built drawings (if included in scope) | Contractor | Within 30 days of completion | ☐ Pending ☐ Submitted |
| 10 | Permit and inspection documentation | Contractor / AHJ | At completion | ☐ Pending ☐ On File |

📅  SECTION 6 — PROJECT SCHEDULE & MILESTONES

The following schedule reflects estimated durations based on normal site conditions and material availability. Delays caused by Client, weather, permit hold, or supply chain disruptions may require schedule revision via written notice.

| # | Milestone / Phase | Planned Start | Planned End | Est. Duration | Responsible | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Client Authorization & Deposit Received | __________ | __________ | — | Client | ☐ |
| 2 | Equipment Order Placed | __________ | __________ | — | Contractor | ☐ |
| 3 | Permit Application Submitted | __________ | __________ | 1–5 days | Contractor | ☐ |
| 4 | Permit Issued | __________ | __________ | Varies | AHJ | ☐ |
| 5 | Equipment Delivery to Site | __________ | __________ | Varies | Supplier | ☐ |
| 6 | Site Preparation / Demo / Removal | __________ | __________ | ___ days | Contractor | ☐ |
| 7 | Rough-In — Ductwork & Piping | __________ | __________ | ___ days | Contractor | ☐ |
| 8 | Rough-In — Electrical / Controls | __________ | __________ | ___ days | Elec. Sub | ☐ |
| 9 | Equipment Set & Installation | __________ | __________ | ___ days | Contractor | ☐ |
| 10 | Refrigerant Piping & Charge | __________ | __________ | ___ days | Contractor | ☐ |
| 11 | Controls Wiring & Programming | __________ | __________ | ___ days | Contractor | ☐ |
| 12 | Start-Up & Commissioning | __________ | __________ | ___ days | Contractor | ☐ |
| 13 | AHJ Inspection | __________ | __________ | Varies | AHJ | ☐ |
| 14 | Owner Training & Turnover | __________ | __________ | 1 day | Contractor | ☐ |
| 15 | Final Punchlist & Closeout | __________ | __________ | ___ days | Contractor | ☐ |

6.1  Contractor shall provide written notice to Client if any milestone is expected to be delayed by more than [2] business days from the planned date.

6.2  Client shall provide required access, approvals, and decisions within [2] business days of request to avoid schedule impact.

6.3  Equipment lead times are subject to manufacturer availability. Contractor is not responsible for manufacturer delays. Schedule will be revised upon confirmed delivery date.

💰  SECTION 7 — PRICING, PAYMENT SCHEDULE & CHANGE ORDERS

7.1  DETAILED COST BREAKDOWN

| # | Line Item Description | Unit | Qty | Unit Price | Extended Price |
| --- | --- | --- | --- | --- | --- |
| EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Primary HVAC Equipment — [Description] | EA | 1 | $__________ | $__________ |
| 2 | Secondary / Supplemental Equipment | EA |  | $__________ | $__________ |
| 3 | Accessories / Ancillary Equipment | LS | 1 | $__________ | $__________ |
| LABOR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Installation Labor — Equipment | Hrs | ____ | $______/hr | $__________ |
| 5 | Installation Labor — Ductwork | Hrs | ____ | $______/hr | $__________ |
| 6 | Installation Labor — Refrigerant Piping | Hrs | ____ | $______/hr | $__________ |
| 7 | Installation Labor — Electrical / Controls | Hrs | ____ | $______/hr | $__________ |
| 8 | Start-Up, Commissioning & Training | Hrs | ____ | $______/hr | $__________ |
| MATERIALS & CONSUMABLES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Sheet Metal Ductwork & Fittings | LS | 1 | $__________ | $__________ |
| 10 | Flexible Duct & Insulation | LF | ____ | $______/LF | $__________ |
| 11 | Refrigerant Piping (Copper ACR) | LF | ____ | $______/LF | $__________ |
| 12 | Refrigerant — [Type] (___ lbs) | LB | ____ | $______/lb | $__________ |
| 13 | Electrical Materials & Control Wiring | LS | 1 | $__________ | $__________ |
| 14 | Miscellaneous Hardware & Consumables | LS | 1 | $__________ | $__________ |
| PERMIT, SUBCONTRACTOR & OTHER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Permit & Inspection Fees (est.) | LS | 1 | $__________ | $__________ |
| 16 | Electrical Sub-Contractor | LS | 1 | $__________ | $__________ |
| 17 | TAB / Balancing (if included) | LS | 1 | $__________ | $__________ |
| SUBTOTAL (before tax): |  |  |  |  | $__________ |
| Sales Tax (___%): |  |  |  |  | $__________ |
| TOTAL CONTRACT PRICE: |  |  |  |  | $__________ |

7.2  PAYMENT SCHEDULE

| # | Payment Milestone | % of Total | Amount Due | Due Date / Trigger |
| --- | --- | --- | --- | --- |
| 1 | Deposit — Upon Authorization & Execution | [30]% | $__________ | Upon signing |
| 2 | Progress Payment — Equipment Delivered to Site | [30]% | $__________ | Delivery confirmed |
| 3 | Progress Payment — Rough-In Complete | [20]% | $__________ | Prior to startup |
| 4 | Final Payment — Substantial Completion | [20]% | $__________ | Upon system operation |

7.3  CHANGE ORDER PROCEDURE

7.3.1  Any work beyond the scope defined in Section 3 requires a written Change Order (CO) signed by both parties before work commences.

7.3.2  Contractor shall submit a written CO proposal within [2] business days of identifying a scope change. CO shall describe the additional work, time impact, and price adjustment.

7.3.3  Verbal authorizations for additional work must be confirmed in writing within [24] hours. Contractor may suspend additional work pending written authorization.

7.3.4  Change orders that increase contract value by more than [10]% may require a revised payment schedule to be negotiated.

7.3.5  Time and material (T&M) change orders shall be documented with daily field reports signed by Client's representative.

✅  SECTION 8 — QUALITY CONTROL & INSPECTIONS

| # | Inspection / QC Checkpoint | Responsible Party | Timing | Sign-Off Required |
| --- | --- | --- | --- | --- |
| 1 | Duct rough-in — size, routing, and insulation | Contractor | Before cover-up | ☐ Contractor ☐ Client |
| 2 | Refrigerant piping — leak test (nitrogen) | Contractor | Before insulation | ☐ Contractor |
| 3 | Refrigerant system — evacuation to <500 microns | Contractor | Before charge | ☐ Contractor |
| 4 | Electrical — continuity and polarity check | Elec. Sub / Contractor | Before energize | ☐ Elec. Sub |
| 5 | Equipment set — level, clearance, and anchorage | Contractor | Before start-up | ☐ Contractor |
| 6 | Controls wiring — point-to-point verification | Contractor | Before start-up | ☐ Contractor |
| 7 | Manufacturer start-up checklist | Contractor / Mfr. | At startup | ☐ Contractor ☐ Mfr. |
| 8 | AHJ rough-in inspection (if required) | AHJ / Contractor | Per permit schedule | ☐ AHJ |
| 9 | AHJ final inspection and sign-off | AHJ / Contractor | At completion | ☐ AHJ |
| 10 | Client walkthrough and acceptance | Client / Contractor | At completion | ☐ Client ☐ Contractor |

✍️  SECTION 9 — AUTHORIZATION TO PROCEED

BY SIGNING BELOW, Client authorizes Contractor to proceed with the work described in this Scope of Work at the price and schedule identified herein. Client confirms they have reviewed and understood all sections of this document, including the scope, exclusions, payment terms, and change order procedure.

| CONTRACTOR | CLIENT — AUTHORIZED TO PROCEED |
| --- | --- |
| Authorized Signature: ____________________________ | Authorized Signature: ____________________________ |
| Printed Name: ____________________________ | Printed Name: ____________________________ |
| Title: ____________________________ | Title: ____________________________ |
| Date: ____________________________ | Date Authorized: ____________________________ |
| SOW Version / Revision: ____________________________ | Deposit Amount Tendered: ____________________________ |

Exhibits Incorporated by Reference:  Exhibit A — Equipment Cut Sheets  |  Exhibit B — Change Order Form  |  Exhibit C — Start-Up Report  |  Exhibit D — O&M Manuals

This document is a template for use by qualified HVAC contractors. Consult a licensed attorney before use. [Your HVAC Co.] makes no warranty as to the legal sufficiency of this template in any jurisdiction.