HVAC SERVICE CALL CHECKLIST Complete all applicable sections. Document existing conditions, measurements, repairs performed, and recommendations before leaving the job site.

📋  SECTION 1 — SERVICE CALL & CUSTOMER INFORMATION

| Work Order #: ________________________ | Service Date: ________________________ |
| --- | --- |
| Customer Name: ________________________ | Customer Phone: ________________________ |
| Service Address: ________________________ | City / State / ZIP: ________________________ |
| Contact On-Site: ________________________ | Contact Phone: ________________________ |
| Technician Name: ________________________ | Technician ID / License: ________________________ |
| Dispatch Time: ________________________ | On-Site Arrival Time: ________________________ |
| Call Priority: ________________________ | Service Agreement #: ________________________ |
| PO # (if required): ________________________ | Invoice #: ________________________ |

🏷️  SECTION 2 — EQUIPMENT IDENTIFICATION

| Equipment Type: ________________________ | Equipment Location: ________________________ |
| --- | --- |
| Manufacturer: ________________________ | Model Number: ________________________ |
| Serial Number: ________________________ | Install Year (approx): ________________________ |
| Refrigerant Type: ________________________ | Nominal Capacity: ________________________ |
| Voltage / Phase / Hz: ________________________ | Controls / Thermostat: ________________________ |
| Last Service Date: ________________________ | Last Service Tech: ________________________ |

💬  SECTION 3 — CUSTOMER COMPLAINT & REPORTED SYMPTOMS

ℹ️ NOTE: Check all symptoms reported by the customer. Use the description box to capture their exact words. Do not edit or interpret — record verbatim.

| ☐ | Reported Symptom | ☐ | Reported Symptom |
| --- | --- | --- | --- |
| ☐ | No cooling | ☐ | No heating |
| ☐ | Insufficient cooling / warm | ☐ | Insufficient heating / cold |
| ☐ | Unit not turning on | ☐ | Unit short-cycling / rapid on-off |
| ☐ | Ice on coil / lines | ☐ | Water / condensate leak |
| ☐ | Unusual noise — describe | ☐ | Unusual odor — describe |
| ☐ | High electric bill | ☐ | Thermostat / controls issue |
| ☐ | Tripped breaker / no power | ☐ | Blower not operating |
| ☐ | Compressor not running | ☐ | Outdoor unit not running |
| ☐ | Poor airflow — weak | ☐ | Uneven temps / hot-cold spots |
| ☐ | Refrigerant leak suspected | ☐ | Other — describe in notes |

Customer Description (in their words):

🛡️  SECTION 4 — SAFETY & SITE ASSESSMENT

⚠️  Complete safety assessment BEFORE beginning any diagnostic or service work. Do not proceed if hazards cannot be controlled.

| Task / Inspection Item | Criteria / Action | Pass | Fail | N/A | Measured Value / Notes |
| --- | --- | --- | --- | --- | --- |
| PPE Donned | Safety glasses, gloves, and arc-flash PPE appropriate to task | ☐ | ☐ | ☐ |  |
| Electrical Hazards Assessed | Panel, disconnect, and wiring condition checked before work | ☐ | ☐ | ☐ |  |
| Gas Hazards Assessed | Gas odor or leak check performed; CO detector used if gas unit | ☐ | ☐ | ☐ |  |
| Refrigerant Safety | EPA 608 cert valid; recovery equipment available if needed | ☐ | ☐ | ☐ |  |
| Area Safe for Work | No water, slip, or fall hazards; clearance adequate for access | ☐ | ☐ | ☐ |  |
| LOTO Applied (if service) | Disconnect locked/tagged out before opening electrical panels | ☐ | ☐ | ☐ |  |
| Condensate Safety | No overflow or water damage risk to structure below unit | ☐ | ☐ | ☐ |  |

🔍  SECTION 5 — VISUAL INSPECTION & AS-FOUND CONDITIONS

📌 NOTE: Document as-found conditions objectively. These findings establish the baseline for diagnosis, customer communication, and any warranty or liability considerations.

| Task / Inspection Item | Criteria / Action | Pass | Fail | N/A | Measured Value / Notes |
| --- | --- | --- | --- | --- | --- |
| Thermostat / Controller | Mode, setpoints, schedule, and operation verified as-found | ☐ | ☐ | ☐ |  |
| Filter Condition | Filter inspected: clean / dirty / missing / wrong size / clogged | ☐ | ☐ | ☐ |  |
| Indoor Coil / AHU | Evaporator coil condition: clean / fouled / iced / damaged | ☐ | ☐ | ☐ |  |
| Blower / Air Handler | Blower wheel condition; belt tension/condition if belt-drive | ☐ | ☐ | ☐ |  |
| Outdoor Unit / Condenser | Condenser coil, cabinet, fan blades, and lineset condition | ☐ | ☐ | ☐ |  |
| Refrigerant Lines | Suction line insulation intact; liquid line condition; no oil staining | ☐ | ☐ | ☐ |  |
| Drain Pan & Condensate | Drain pan clear; condensate flowing; no biological growth | ☐ | ☐ | ☐ |  |
| Electrical Panel / Disconnect | No tripped breakers, burnt wires, corrosion, or overheating signs | ☐ | ☐ | ☐ |  |
| Control Board / Wiring | No burnt components, loose terminals, or damaged wiring visible | ☐ | ☐ | ☐ |  |
| Flue / Venting (Gas Units) | Vent pipe clear, connected, and no signs of spillage or blockage | ☐ | ☐ | ☐ |  |
| Gas Train (Gas Units) | Gas valve, manifold, and burners visible condition checked | ☐ | ☐ | ☐ |  |
| Structural / Mounting | Unit mounting, curb, and ductwork connections intact | ☐ | ☐ | ☐ |  |

📊  SECTION 6 — DIAGNOSTIC MEASUREMENTS (AS-FOUND)

ℹ️ NOTE: Record all measurements as-found before any repairs are made. These are the baseline readings that substantiate the diagnosis and justify the repair.

| Parameter | Measured | Unit | Spec / Range | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| Space Temperature — Actual |  | °F | Per thermostat setpoint |  |  |
| Thermostat Setpoint — Cooling |  | °F | Record as-found |  |  |
| Thermostat Setpoint — Heating |  | °F | Record as-found |  |  |
| Supply Air Temp (at register) |  | °F | Record actual |  |  |
| Return Air Temp |  | °F | Record actual |  |  |
| Delta-T (RAT − SAT) |  | °F | 16–22 °F (cooling) |  |  |
| Supply Air CFM (if measurable) |  | CFM | Per design |  |  |
| Static Pressure — Supply |  | in. w.c. | Per design |  |  |
| Static Pressure — Return |  | in. w.c. | Per design |  |  |
| Suction Pressure (as-found) |  | PSIG | Per PT chart |  |  |
| Discharge Pressure (as-found) |  | PSIG | Per PT chart |  |  |
| Superheat (as-found) |  | °F | Per mfr spec |  |  |
| Subcooling (as-found) |  | °F | Per mfr spec |  |  |
| Supply Voltage — L1 to L2 |  | VAC | ±5% nominal |  |  |
| Compressor Amp Draw (as-found) |  | Amps | ≤ nameplate RLA |  |  |
| Blower Amp Draw (as-found) |  | Amps | ≤ nameplate FLA |  |  |
| Control Circuit Voltage |  | VAC | 24 VAC ± 10% |  |  |
| CO Level — Space (gas units) |  | ppm | < 35 ppm (OSHA) |  |  |
| CO Level — Flue (gas units) |  | ppm | < 400 ppm air-free |  |  |

🧠  SECTION 7 — DIAGNOSIS, ROOT CAUSE & FAULT CODES

Diagnosis & Repair Summary

| Customer Complaint | Root Cause Identified | Repair / Action Taken | Parts / Materials | Status |
| --- | --- | --- | --- | --- |

Fault / Error Code Log

| Fault Code | Source | Description | Action Taken |
| --- | --- | --- | --- |

🔧  SECTION 8 — REPAIRS PERFORMED

| Task / Inspection Item | Criteria / Action | Pass | Fail | N/A | Measured Value / Notes |
| --- | --- | --- | --- | --- | --- |
| Filter Replaced | New filter installed: MERV rating _____ ; size _____________ | ☐ | ☐ | ☐ |  |
| Coil Cleaned — Evaporator | Evaporator coil cleaned with approved coil cleaner; rinsed | ☐ | ☐ | ☐ |  |
| Coil Cleaned — Condenser | Condenser coil cleaned; fins straightened where possible | ☐ | ☐ | ☐ |  |
| Blower Cleaned | Blower wheel cleaned of lint, dirt, and debris | ☐ | ☐ | ☐ |  |
| Drain Pan Cleaned | Drain pan cleaned; biological growth treated with algaecide tab | ☐ | ☐ | ☐ |  |
| Condensate Line Cleared | Primary drain flushed; trap cleaned and primed | ☐ | ☐ | ☐ |  |
| Refrigerant Added / Recovered | Charge adjusted — see refrigerant log (Section 9) | ☐ | ☐ | ☐ |  |
| Leak Located & Repaired | Leak location: _________________ ; repair method: _________ | ☐ | ☐ | ☐ |  |
| Contactor Replaced | Failed contactor removed and replaced with OEM-equivalent part | ☐ | ☐ | ☐ |  |
| Capacitor Replaced | Run / start capacitor replaced; new µF rating verified on label | ☐ | ☐ | ☐ |  |
| Control Board Replaced | Failed control board replaced; DIP switches / jumpers set per mfr | ☐ | ☐ | ☐ |  |
| TXV / Metering Device | Metering device inspected or replaced; bulb re-secured | ☐ | ☐ | ☐ |  |
| Reversing Valve (HP) | Reversing valve operation tested; replaced if stuck or leaking | ☐ | ☐ | ☐ |  |
| Belt & Pulleys | Drive belt replaced; pulley alignment and belt tension set | ☐ | ☐ | ☐ |  |
| Bearings / Lubricated | Blower or motor bearings lubricated or replaced as needed | ☐ | ☐ | ☐ |  |
| Electrical Repair | Describe: ________________________________________________ | ☐ | ☐ | ☐ |  |
| Gas Valve / Igniter | Gas valve or igniter replaced; ignition sequence verified | ☐ | ☐ | ☐ |  |
| Flue / Vent Repair | Vent obstruction cleared or vent pipe connection repaired | ☐ | ☐ | ☐ |  |
| Thermostat / Controls | Thermostat or controls reprogrammed / replaced / re-wired | ☐ | ☐ | ☐ |  |
| Other Repair | Describe: ________________________________________________ | ☐ | ☐ | ☐ |  |

♻️  SECTION 9 — REFRIGERANT LOG (Complete if Refrigerant Handled)

⚠️  All refrigerant handling must comply with EPA Section 608. This log is legally required whenever refrigerant is added, recovered, or reclaimed.

| Action | Refrigerant Type | Amount (lbs) | Cylinder / Tank # | Technician & Date |
| --- | --- | --- | --- | --- |
| Recovered from system |  |  |  |  |
| Added to system |  |  |  |  |
| Net charge change (±) |  |  |  |  |
| Reason for charge change |  |  |  |  |

✅  SECTION 10 — POST-REPAIR MEASUREMENTS (AS-LEFT)

📌 NOTE: Record as-left measurements after all repairs are complete and system has reached steady state (minimum 15 minutes). Compare to as-found readings in Section 6 to confirm repair effectiveness.

| Parameter | Measured | Unit | Spec / Range | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| Space Temperature — As-Left |  | °F | At or near setpoint |  |  |
| Supply Air Temp (as-left) |  | °F | Record actual |  |  |
| Return Air Temp (as-left) |  | °F | Record actual |  |  |
| Delta-T (as-left) |  | °F | 16–22 °F (cooling) |  |  |
| Suction Pressure (as-left) |  | PSIG | Per PT chart |  |  |
| Discharge Pressure (as-left) |  | PSIG | Per PT chart |  |  |
| Superheat (as-left) |  | °F | Per mfr spec |  |  |
| Subcooling (as-left) |  | °F | Per mfr spec |  |  |
| Compressor Amp Draw (as-left) |  | Amps | ≤ nameplate RLA |  |  |
| Blower Amp Draw (as-left) |  | Amps | ≤ nameplate FLA |  |  |
| Supply Voltage (as-left) |  | VAC | ±5% nominal |  |  |
| CO Level — Space (as-left) |  | ppm | < 35 ppm (OSHA) |  |  |
| System Run-Time Verified |  | Minutes | ≥ 15 min steady state |  |  |

🔩  SECTION 11 — PARTS & MATERIALS USED

| # | Part / Material Description | Part Number | Qty | Unit | Notes / Source |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |

📌  SECTION 12 — RECOMMENDATIONS & DEFERRED WORK

| 🔴 IMMEDIATE — Safety risk | 🟠 URGENT — Within 30 days | 🔵 PLANNED — Next service | 🟢 MONITOR — Watch only |
| --- | --- | --- | --- |

| Recommendation / Deferred Repair | Urgency | Est. Cost | Customer Response / Notes |
| --- | --- | --- | --- |

⚠️  Immediate (🔴) safety items must be disclosed in writing to the customer. Document customer acknowledgment in the notes column and on the signed work order.

⏱️  SECTION 13 — TIME LOG

| Activity | Arrival / Start | Departure / End | Total Time | Notes |
| --- | --- | --- | --- | --- |
| On-site arrival |  |  |  |  |
| Diagnostic / assessment |  |  |  |  |
| Repair / service work |  |  |  |  |
| Parts run / off-site |  |  |  |  |
| Return & completion |  |  |  |  |

| Total Billable Hours: ____________________ | Travel Time (if billable): ____________________ |
| --- | --- |
| After-Hours / OT Hours: ____________________ | Number of Return Trips: ____________________ |

🏁  SECTION 14 — POST-SERVICE COMPLETION CHECKLIST

| Task / Inspection Item | Criteria / Action | Pass | Fail | N/A | Measured Value / Notes |
| --- | --- | --- | --- | --- | --- |
| System Operational — Verified | Unit confirmed running in correct mode before departing site | ☐ | ☐ | ☐ |  |
| Setpoints Restored / Confirmed | Thermostat setpoints confirmed matching customer preferences | ☐ | ☐ | ☐ |  |
| All Panels Reinstalled | All access panels, covers, and screws reinstalled and secured | ☐ | ☐ | ☐ |  |
| Area Cleaned Up | Work area cleaned; packaging, old parts, and debris removed | ☐ | ☐ | ☐ |  |
| Refrigerant Log Complete | EPA refrigerant log completed if any refrigerant was handled | ☐ | ☐ | ☐ |  |
| Photo Documentation | Before/after photos taken; uploaded to job record | ☐ | ☐ | ☐ |  |
| Customer Walkthrough | Technician walked customer through findings and repairs performed | ☐ | ☐ | ☐ |  |
| Recommendations Explained | All deferred work and recommendations explained to customer | ☐ | ☐ | ☐ |  |
| Safety Hazards Disclosed | Any unresolved safety issues clearly communicated to customer | ☐ | ☐ | ☐ |  |
| Service Agreement Reviewed | Service agreement status reviewed; renewal offered if applicable | ☐ | ☐ | ☐ |  |
| Work Order Signed | Customer or authorized representative signed work order | ☐ | ☐ | ☐ |  |
| Invoice / Receipt Provided | Copy of invoice or receipt provided to customer on departure | ☐ | ☐ | ☐ |  |

📝  SECTION 15 — FIELD NOTES & TECHNICAL OBSERVATIONS

FIELD NOTES & TECHNICAL OBSERVATIONS

✍️  SECTION 16 — SERVICE COMPLETION & CUSTOMER ACCEPTANCE

| Lead Technician | Dispatcher / Supervisor | Customer / Authorized Rep |
| --- | --- | --- |
| Signature: _____________________ | Signature: _____________________ | Signature: _____________________ |
| Printed Name: _____________________ | Printed Name: _____________________ | Printed Name: _____________________ |
| Company: _____________________ | Company: _____________________ | Company: _____________________ |
| Date: _____________________ | Date: _____________________ | Date: _____________________ |

Customer signature confirms work was performed as described. A copy of this service record shall be provided to the customer upon request.