HVAC SERVICE ORDER Complete all sections. Obtain customer signature upon job completion. Retain original for billing.

📋  SECTION 1 — JOB & CUSTOMER INFORMATION

| Service Order #: ________________________ | Service Date: ________________________ |
| --- | --- |
| Customer / Company Name: ________________________ | Primary Phone: ________________________ |
| Service Address: ________________________ | City, State, ZIP: ________________________ |
| Billing Address (if different): ________________________ | Email: ________________________ |
| Site Contact Name: ________________________ | Alt / Cell Phone: ________________________ |
| Purchase Order #: ________________________ | Contract / Agreement #: ________________________ |

🔧  SECTION 2 — SYSTEM / EQUIPMENT INFORMATION

| Equipment Type: ________________________ | Make / Manufacturer: ________________________ |
| --- | --- |
| Model Number: ________________________ | Serial Number: ________________________ |
| Unit Location / ID: ________________________ | Refrigerant Type: ________________________ |
| Nominal Tonnage / Capacity: ________________________ | Voltage / Phase: ________________________ |
| Install Date: ________________________ | Last Service Date: ________________________ |
| Warranty Expiration: ________________________ | Service Contract: ________________________ |

📝  SECTION 3 — SERVICE TYPE & PRIORITY

| [ ] Emergency / No Cooling | [ ] Preventive Maintenance | [ ] Diagnostic / Inspection | [ ] Startup / Commissioning |
| --- | --- | --- | --- |
| [ ] Repair / Corrective | [ ] Refrigerant Service | [ ] Equipment Replacement | [ ] Warranty Call |

Customer Complaint / Reported Problem:

📊  SECTION 4 — SYSTEM PERFORMANCE MEASUREMENTS

Refrigerant & Pressure Data

| Parameter | Measured | Unit | Spec / Range | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| Suction Pressure (PSI) |  | PSIG |  | ☐ OK ☐ Adj |  |
| Discharge Pressure (PSI) |  | PSIG |  | ☐ OK ☐ Adj |  |
| Superheat (SH) |  | °F | 8–18°F | ☐ OK ☐ Adj |  |
| Subcooling (SC) |  | °F | 10–18°F | ☐ OK ☐ Adj |  |
| Suction Line Temp |  | °F |  | ☐ OK ☐ Adj |  |
| Liquid Line Temp |  | °F |  | ☐ OK ☐ Adj |  |
| Refrigerant Added / Recovered |  | lbs | N/A | ☐ Added ☐ Recov |  |

Electrical Measurements

| Parameter | Measured | Unit | Spec / Range | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| Supply Voltage L1-L2 |  | VAC | ±10% Rated | ☐ OK ☐ Flag |  |
| Supply Voltage L1-L3 |  | VAC | ±10% Rated | ☐ OK ☐ Flag |  |
| Compressor Amps (RLA) |  | A |  | ☐ OK ☐ Flag |  |
| Condenser Fan Amps |  | A |  | ☐ OK ☐ Flag |  |
| Evap / Blower Fan Amps |  | A |  | ☐ OK ☐ Flag |  |
| Capacitor (µF) — Run |  | µF | ±6% Rated | ☐ OK ☐ Repl |  |
| Capacitor (µF) — Start |  | µF | ±20% Rated | ☐ OK ☐ Repl |  |

Airflow & Temperature Data

| Parameter | Measured | Unit | Spec / Range | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| Supply Air Temp |  | °F |  | ☐ OK ☐ Flag |  |
| Return Air Temp |  | °F |  | ☐ OK ☐ Flag |  |
| Delta T (ΔT) |  | °F | 16–22°F | ☐ OK ☐ Flag |  |
| Supply Static Pressure |  | in. w.c. |  | ☐ OK ☐ Flag |  |
| Return Static Pressure |  | in. w.c. |  | ☐ OK ☐ Flag |  |
| Total External Static |  | in. w.c. |  | ☐ OK ☐ Flag |  |
| Filter Pressure Drop |  | in. w.c. |  | ☐ OK ☐ Flag |  |

✅  SECTION 5 — INSPECTION CHECKLIST

Refrigeration & Mechanical

| Inspection Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Checked for refrigerant leaks (electronic / bubble test) | ☐ | ☐ | ☐ | ☐ |
| Inspected refrigerant lines — insulation, support, routing | ☐ | ☐ | ☐ | ☐ |
| Verified TXV / metering device operation | ☐ | ☐ | ☐ | ☐ |
| Inspected compressor contactor — pitting, wear, voltage drop | ☐ | ☐ | ☐ | ☐ |
| Checked crankcase heater operation | ☐ | ☐ | ☐ | ☐ |
| Inspected reversing valve (heat pump — if applicable) | ☐ | ☐ | ☐ | ☐ |
| Verified low-ambient / head pressure controls | ☐ | ☐ | ☐ | ☐ |
| Checked economizer damper operation and blade seals | ☐ | ☐ | ☐ | ☐ |

Electrical & Controls

| Inspection Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspected all wiring — no fraying, burns, or loose terminals | ☐ | ☐ | ☐ | ☐ |
| Checked disconnect / fusing — correct size, properly rated | ☐ | ☐ | ☐ | ☐ |
| Verified thermostat / BAS setpoints and calibration | ☐ | ☐ | ☐ | ☐ |
| Tested all safeties: HP, LP, freeze stat, rollout, flame | ☐ | ☐ | ☐ | ☐ |
| Inspected control board — no fault codes, burn marks, corrosion | ☐ | ☐ | ☐ | ☐ |
| Verified defrost board and timer (heat pump — if applicable) | ☐ | ☐ | ☐ | ☐ |
| Tested emergency heat / auxiliary strip heat | ☐ | ☐ | ☐ | ☐ |
| Checked smoke detector and UV light (if installed) | ☐ | ☐ | ☐ | ☐ |

Air Distribution & Filtration

| Inspection Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Replaced / inspected air filter — recorded size and MERV | ☐ | ☐ | ☐ | ☐ |
| Cleaned evaporator coil (if accessible) — inspected for damage | ☐ | ☐ | ☐ | ☐ |
| Cleaned condenser coil — removed debris, checked fins | ☐ | ☐ | ☐ | ☐ |
| Inspected blower wheel and housing for buildup | ☐ | ☐ | ☐ | ☐ |
| Checked belt tension and sheave alignment (if belt-driven) | ☐ | ☐ | ☐ | ☐ |
| Verified all supply / return grilles and registers are open | ☐ | ☐ | ☐ | ☐ |
| Checked condensate drain, pan, and float switch | ☐ | ☐ | ☐ | ☐ |
| Flushed condensate drain with approved cleaner | ☐ | ☐ | ☐ | ☐ |

Safety & Compliance

| Inspection Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verified refrigerant type and quantity on equipment label | ☐ | ☐ | ☐ | ☐ |
| Confirmed 608 refrigerant tracking log updated | ☐ | ☐ | ☐ | ☐ |
| Checked unit for unusual noise, vibration, or odors | ☐ | ☐ | ☐ | ☐ |
| Confirmed proper clearances around unit / service access | ☐ | ☐ | ☐ | ☐ |
| Verified lockout / tagout procedures followed | ☐ | ☐ | ☐ | ☐ |
| Left work area clean — debris removed, panels secured | ☐ | ☐ | ☐ | ☐ |
| Photographed any deficiencies noted during service | ☐ | ☐ | ☐ | ☐ |
| Customer informed of any deferred repairs or recommendations | ☐ | ☐ | ☐ | ☐ |

🔍  SECTION 6 — SYSTEM CONDITION ASSESSMENT

| Assessment Item | Status A | Status B | Status C | Status D |
| --- | --- | --- | --- | --- |
| Overall System Condition | ☐ Excellent | ☐ Good | ☐ Fair | ☐ Poor |
| Refrigerant Status | ☐ Correct | ☐ Low | ☐ Overcharged | ☐ Recovered |
| Filter Condition | ☐ New | ☐ Clean | ☐ Dirty — Replaced | ☐ N/A |
| Electrical Safety | ☐ Pass | ☐ Fail | ☐ Advisory | ☐ N/A |
| Warranty Status | ☐ Active | ☐ Expired | ☐ Voided | ☐ Unknown |

Deficiencies, Findings & Recommendations:

🔩  SECTION 7 — PARTS & MATERIALS USED

| Part / Material Description | Part # | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Parts / Materials Subtotal: |  |  |  | $__________ |

👷  SECTION 8 — LABOR & TIME LOG

| Technician | Task / Description | Start | End | Hours | Rate | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Labor Subtotal: |  |  |  |  |  | $__________ |

💲  SECTION 9 — INVOICE SUMMARY

|  | Parts / Materials Subtotal:; $__________. Labor Subtotal:; $__________. Trip / Diagnostic Charge:; $__________. Applicable Tax:; $__________. Discount / Credit:; ($__________). TOTAL DUE:; $__________. |
| --- | --- |

Billing Notes / Payment Terms:

✍️  SECTION 10 — SIGNATURES & AUTHORIZATION

By signing below, the customer acknowledges that described services were performed satisfactorily and authorizes payment per agreed terms.

| Technician Signature | Customer Signature | Customer Print Name |
| --- | --- | --- |

| Date Completed: _______________ | Authorization #: _______________ | Payment Method: _______________ |
| --- | --- | --- |

OFFICE USE ONLY  |  Invoiced: ☐     Paid: ☐     Filed: ☐     Date: ___________     By: ___________